SCOPE 1 Fugitive emission:

| Refrigerant | GWP (100-year) |
| --- | --- |
| R-22 | 1810 |
| R-32 | 675 |
| R-134a | 1430 |
| R-404A | 3922 |
| R-407C | 1774 |
| R-410A | 2088 |
| R-507A | 3985 |
| R-508B | 13396 |
| HFC-227ea | 3220 |
| FK-5-1-12 (Novec 1230) | ~1 |

SCOPE 1 DG Set:Stationary combustion

Diesel = 2.68 kg CO2/L

SCOPE 1 Campus owned vehicle: Mobile combustion

| Year | Petrol (kg CO₂/L) | Diesel (kg CO₂/L) | EV (kg CO₂/kWh) |
| --- | --- | --- | --- |
| 2020 | 2.31 | 2.68 | 0.82 |
| 2021 | 2.31 | 2.68 | 0.79 |
| 2022 | 2.31 | 2.68 | 0.76 |
| 2023 | 2.31 | 2.68 | 0.73 |
| 2024 | 2.31 | 2.68 | 0.71 |
| 2025* | 2.31 | 2.68 | 0.70 (est.) |

SCOPE 1 LPG CONSUMPTION:

Diesel kg     2.68 CO2e/Kg

SCOPE 2 ELECTRICITY tCO2/MW/h

2020- 0.83

2021- 0.79

2022 - 0.70

2023 - 0.81

2024 - 0.710

SCOPE 3 GARDEN WASTE  CO2 equivalent

Composting                       0.02

Landfill                               0.6

Open burning                    1.5

Mulching                          0.002

SCOPE 3 FOOD WASTE BIOGAS

| Volatile solids fraction of food waste | 0.85 (0.80–0.90) | tonne VS / tonne waste |
| --- | --- | --- |
| Maximum methane generation potential | 0.6 | m³ CH₄ / kg VS |
| Methane correction factor (controlled anaerobic digestion system) | 1 | dimensionless |
| Methane capture efficiency of digester | 0.8 – 0.95 (commonly 0.9) | fraction |
| Density of methane at standard temperature and pressure | 0.67 | kg CH₄ / m³ CH₄ |
| Global warming potential of methane | 28 | kg CO₂e / kg CH₄ |

SCOPE 3 STP

EF aerobic =0.25kg CO₂e/m³

EF anaerobic = 2.0kg CO₂e/m³

SCOPE 3 EMBODIED EMISSION

Area based embodied carbon method -  0.2 Kg/CO2/m2/year

SCOPE 3 BIOGENIC

Biogenics = Kg/ CO2/per person/ per day